How to upgrade your IBM PC JX to the latest Bios version.

The early version of the IBM ROM bios in the IBM PC JX only supported the 3.5” drives in 360Kb mode.  This means that you can only boot from a diskette which is formatted 360Kb.  This is fine if you are only running IBM PC JX DOS 2.1 as it comes on a 3.5” diskette formatted 360Kb.  You can’t however format or read 720Kb diskettes.

IBM Australia released a new version of the IBM PC JX DOS, which is version 3.2.  It came on a 3.5” diskette, formatted 720Kb.  In order to make it readable by the IBM PC JX, IBM also supplied a new version of the ROM BIOS chip.  Once you installed the new ROB Bios chip, the IBM PC JX could boot from the 720Kb 3.2 DOS diskette.  It could then read and format 720Kb diskettes.

If your IBM PC JX doesn’t have the latest IBM Bios version, it is possible to upgrade to the latest Bios version, which has a date code of “05/26/86” and P/N of the Bios ROM is 8629K1 or 8732KY.

The original Bios chip is a 28 pin masked ROM chip.  Unfortunately, it’s not possible to get a pin for pin compatible EEPROM today to replace it.  However, you can use a 32 pin EEPROM like the AM27C010 as a replacement.

| The pin configuration of the original masked ROM chip. | The pin configuration of AM27C010 |
| --- | --- |

In order to replace the IBM JX ROM with the AM27C010, you first have to burn the new Bios image into the AM27C010 with an EEPROM programmer.  Once done, you need to make an adapter in order to use it in the IBM PC JX.  The following steps show how an adapter can be made.

You can also read how to do this on http://www.adsb.co.uk/bbc/adaptor.html

| - Start with one 32 Pin IC socket and one 28 pin socket. Make sure you use the ones with a solid pins and a hole at the top, not the ones with a weak flat blade as they break off very easyily. |  |
| --- | --- |
| - If you don’t have a 28 pin IC socket, you can use another 32 pin one. You would need to cut off from one of the 32 Pin adapters the two top rows, which will make it a 28 pin IC socket. Cut from the marked end. |  |
| - Take the 32 Pin IC socket and cut pins 1,2,24,30,31,and 32 of the 32-pin socket to about half length |  |
| - Solder all the full length pins on the right hand side of the 32 Pin socket to the corresponding buckets on the 28 pin socket (i.e. pins 17 to 23 on the 32-pin socket are connected to pins 15 to 21 on the 28-pin socket, and pins 25 to 29 on the 32-pin socket are connected to pins 23 to 27 on the 28-pin socket). Careful not to bridge any pins with solder blobs! |  |
| - Connect pin 2 of the 32-pin socket to pin 22 of the 28-pin socket. Make sure pin 22 of the 28 pin socket doesn’t touch the pin above it on the 32 pin socket. |  |
| - Connect pin 32 of the 32-pin socket to pin 28 of the 28-pin socket - Connect pins 1 and 24 of the 32-pin socket to pin 16 of the 32-pin socket - Connect pins 1 and 24 of the 32-pin socket to pin 16 of the 32-pin socket |  |
| Use a volt meter on the diode/buzzer setting to check all pins are connected properly and you don’t have any solder bridges between pins. |  |
| You can now install the AM270C10 IC into the 32 pin IC socket. Make sure it is oriented the correct way. |  |
| Carefully remove the existing ROM Bios chip and insert the one you have just made onto the socket on the motherboard. Boot the IBM PC JX into basic and make sure it boots properly. If so, you can install the Diskette drive again on top. |  |
| Despite being taller than the original ROM Bios chip, it can still fit under the diskette drive. |  |
| Install the IBM PC DOS JX 3.2 diskette and boot the IBM PC JX. It should recognise the 720Kb diskette and boot from it. |  |